/************************

Chip Type: ATmega16L

Program type: Application

Clock frequency: 1 MHz

CodeVision AVR style

***********************/

#include <mega16.h>

#include <delay.h>

#include <lcd.h>

#include <string.h>

/* -------------------------------

EEPROM-stored password

- In CodeVision AVR, declaring 'char eeprom X[]' places it in EEPROM.

- We initialize with a default 6-digit code (this value stored in EEPROM at compile time).

--------------------------------*/

#pragma warn -par

char eeprom saved_password[6] = { '1','2','3','4','5','6' };  /* default password */

#pragma warn +par

/* Runtime RAM copies */

char password_entry[6];

char password_temp[6];

unsigned char key_val;            /* last key value from keypad */

unsigned char attempts;           /* number of failed attempts */

bit access_granted;               /* 0/1 flag */

/* Function prototypes */

unsigned char keypad(void);

void delay_debounce(void);

unsigned char password_check(void);

void new_password(void);

void load_password_from_eeprom(void);

void store_password_to_eeprom(void);

/* Helper: map numeric key id to ASCII character for display/storage */

char key_to_char(unsigned char key)

{

switch(key) {

case 1: return '1';

case 2: return '2';

case 3: return '3';

case 4: return '4';

case 5: return '5';

case 6: return '6';

case 7: return '7';

case 8: return '8';

case 9: return '9';

case 10: return '';   / row4 col0 */

case 0: return '0';    /* row4 col1 */

case 11: return '#';   /* row4 col2 */

default: return 0;

}

}

/* ------------------------------

Main

------------------------------*/

void main(void)

{

/* PORT configuration based on your schematic:

- PORTB used for LCD data (DB0..DB7)

- PORTC used as outputs for LEDs, buzzer, relay (PC0..PC3)

- PORTD:

PD0..PD2 = keypad columns (inputs with pull-ups)

PD3..PD6 = keypad rows (outputs driven low/high)

PD7 unused or general input (we keep as input)

*/

DDRB = 0x00;   /* LCD data as input to start; lcd_init will handle */

PORTB = 0x00;

DDRC = 0xFF;   /* PC0..PC7 as outputs for LEDs/buzzer/relay */

PORTC = 0x00;   /* all off initially */

DDRD = 0x78;   /* 0b01111000 : PD3..PD6 as outputs (rows), PD0..PD2 inputs (cols) */

/* Enable pull-ups on PD0..PD2 only */

PORTD = 0x07;  /* 0b00000111 : pull-ups enabled on PD0..PD2, rows idle high */

/* Init LCD */

lcd_init(16);

lcd_clear();

/* load password from EEPROM (copy to RAM area if desired) */

load_password_from_eeprom();

attempts = 0;

access_granted = 0;

/* Main menu loop */

while (1)

{

/* Show simple menu:

*  press '*' to enter (open)  -> star mapped to key 10 in mapping

*  press '#' to change password -> hash mapped to key 11

*/

lcd_clear();

lcd_putsf("Enter: *  Change: #");

lcd_gotoxy(0,1);

lcd_putsf("Press key...");

/* wait for a key */

unsigned char k = keypad();

if (k == 10) { /* '*' -> enter password mode */

access_granted = password_check();

if (access_granted) {

/* Activate relay and show accepted */

lcd_clear();

lcd_putsf("Pass accepted");

PORTC.1 = 1;    /* relay ON (example: PC1) */

delay_ms(1500);

PORTC.1 = 0;    /* relay OFF */

/* reset attempts */

attempts = 0;

} else {

/* failure handling done inside password_check */

}

} else if (k == 11) { /* '#' -> change password flow */

new_password();

} else {

/* ignore others */

}

delay_ms(300);

}

}

/* ------------------------------

Read EEPROM-stored password into RAM if needed

(for now we keep using saved_password EEPROM directly for comparisons)

------------------------------*/

void load_password_from_eeprom(void)

{

/* In CodeVision the eeprom variable can be read directly, but for

clarity we copy into a RAM buffer if needed.

Here we do nothing because we compare directly with saved_password.

*/

}

/* Write saved_password[] in EEPROM already occurs by assigning saved_password[]

if you want to explicitly write you would use CodeVision EEPROM API.

We'll provide store function for clarity in case multiple writes needed.

*/

void store_password_to_eeprom(void)

{

/* In CodeVision, assigning to eeprom array elements writes them to EEPROM.

So we simply write saved_password[i] = password_temp[i] etc.

Here this function does nothing extra because calling code assigns to saved_password.

If using avr-gcc, replace with eeprom_write_byte(&saved_password[i], byte).

*/

}

/* ------------------------------

password_check(): reads 6 digits from keypad,

compares with saved_password (EEPROM), limits attempts to 3,

sets attempts counter and returns 1 if accepted, 0 otherwise.

------------------------------*/

unsigned char password_check(void)

{

unsigned char i;

unsigned char match_count;

unsigned char k;

lcd_clear();

lcd_putsf("Enter password:");

lcd_gotoxy(0,1);

/* read 6 digits */

for (i = 0; i < 6; i++) {

k = keypad();                      /* wait and return key code */

password_entry[i] = key_to_char(k);

lcd_putchar('*');

}

delay_ms(300);

/* compare with EEPROM saved_password */

match_count = 0;

for (i = 0; i < 6; i++) {

if (password_entry[i] == saved_password[i]) match_count++;

}

if (match_count == 6) {

/* accepted */

lcd_clear();

lcd_putsf("Pass accepted...");

PORTC.1 = 1;    /* relay */

PORTC.0 = 1;    /* buzzer on briefly */

delay_ms(300);

PORTC.0 = 0;

delay_ms(1200);

PORTC.1 = 0;

attempts = 0;

return 1;

} else {

/* not accepted */

attempts++;

lcd_clear();

lcd_putsf("Invalid Pass!");

PORTC.3 = 1; /* Red LED on */

PORTC.0 = 1; /* buzzer short */

delay_ms(500);

PORTC.0 = 0;

PORTC.3 = 0;

if (attempts >= 3) {

/* lockout and alarm */

lcd_clear();

lcd_putsf("Too many tries!");

PORTC.2 = 1; /* persistent alarm LED or similar */

/* sound buzzer for longer */

PORTC.0 = 1;

delay_ms(3000);

PORTC.0 = 0;

/* system locked until reset - simulate by waiting indefinitely */

while (1) {

/* could implement external reset input reading here */

delay_ms(1000);

}

}

return 0;

}

}

/* ------------------------------

new_password(): user enters old password, then new password twice to confirm

------------------------------*/

void new_password(void)

{

unsigned char i, k;

unsigned char ok_old = 0;

unsigned char confirm_ok = 0;

lcd_clear();

lcd_putsf("Change password");

delay_ms(500);

/* ask for old password */

lcd_clear();

lcd_putsf("Enter old pass:");

lcd_gotoxy(0,1);

for (i = 0; i < 6; i++) {

k = keypad();

password_entry[i] = key_to_char(k);

lcd_putchar('*');

}

delay_ms(300);

/* compare */

ok_old = 1;

for (i = 0; i < 6; i++) {

if (password_entry[i] != saved_password[i]) { ok_old = 0; break; }

}

if (!ok_old) {

lcd_clear();

lcd_putsf("Old pass wrong!");

PORTC.0 = 1; delay_ms(300); PORTC.0 = 0;

return;

}

/* get new password */

lcd_clear();

lcd_putsf("Enter new pass:");

lcd_gotoxy(0,1);

for (i = 0; i < 6; i++) {

k = keypad(); password_temp[i] = key_to_char(k); lcd_putchar('*');

}

delay_ms(300);

/* confirm */

lcd_clear();

lcd_putsf("Confirm pass:");

lcd_gotoxy(0,1);

for (i = 0; i < 6; i++) {

k = keypad(); password_entry[i] = key_to_char(k); lcd_putchar('*');

}

delay_ms(300);

confirm_ok = 1;

for (i = 0; i < 6; i++) {

if (password_entry[i] != password_temp[i]) { confirm_ok = 0; break; }

}

if (confirm_ok) {

/* write to EEPROM */

for (i = 0; i < 6; i++) {

saved_password[i] = password_temp[i]; /* in CodeVision this writes to EEPROM */

}

store_password_to_eeprom(); /* placeholder if extra actions needed */

lcd_clear();

lcd_putsf("Pass changed..");

delay_ms(1200);

lcd_clear();

} else {

lcd_clear();

lcd_putsf("Confirm mismatch!");

PORTC.0 = 1; delay_ms(400); PORTC.0 = 0;

delay_ms(400);

}

}

/* ------------------------------

keypad(): scans 4x3 keypad connected as:

Rows -> PD3, PD4, PD5, PD6  (outputs)

Cols -> PD0, PD1, PD2       (inputs with pull-ups)

returns numeric code:

1..9 numbers, 10='*', 0='0', 11='#'

Includes debounce

------------------------------*/

unsigned char keypad(void)

{

unsigned char key = 255; /* invalid */

unsigned char row;

unsigned char col;

unsigned int timeout;

/* Ensure rows are high (inactive) initially */

PORTD |= (1<<3)|(1<<4)|(1<<5)|(1<<6);

while (1) {

/* scan each row by pulling it low and checking columns */

for (row = 0; row < 4; row++) {

/* set all rows high */

PORTD |= (1<<3)|(1<<4)|(1<<5)|(1<<6);

/* pull current row low */

PORTD &= ~((1<<(3+row)));

delay_ms(5); /* tiny settle */

/* read columns */

if ((PIND & 0x01) == 0) { col = 0; }      /* PD0 low */

else if ((PIND & 0x02) == 0) { col = 1; } /* PD1 low */

else if ((PIND & 0x04) == 0) { col = 2; } /* PD2 low */

else { col = 0xFF; }

if (col != 0xFF) {

/* Debounce: wait to confirm stable press */

delay_ms(30);

/* re-check */

if ((row==0 && ((PIND & 0x01)==0 || (PIND & 0x02)==0 || (PIND & 0x04)==0)) ||

(row==1 && ((PIND & 0x01)==0 || (PIND & 0x02)==0 || (PIND & 0x04)==0)) ||

(row==2 && ((PIND & 0x01)==0 || (PIND & 0x02)==0 || (PIND & 0x04)==0)) ||

(row==3 && ((PIND & 0x01)==0 || (PIND & 0x02)==0 || (PIND & 0x04)==0)) )

{

/* map row,col to key */

if (row==0 && col==0) key = 1;

if (row==0 && col==1) key = 2;

if (row==0 && col==2) key = 3;

if (row==1 && col==0) key = 4;

if (row==1 && col==1) key = 5;

if (row==1 && col==2) key = 6;

if (row==2 && col==0) key = 7;

if (row==2 && col==1) key = 8;

if (row==2 && col==2) key = 9;

if (row==3 && col==0) key = 10;  /* '*' */

if (row==3 && col==1) key = 0;   /* '0' */

if (row==3 && col==2) key = 11;  /* '#' */

/* wait for key release before returning */

while (((PIND & 0x07) != 0x07)) { /* any column low */

delay_ms(10);

}

delay_ms(30);

return key;

}

}

} /* end for rows */

} /* end while */

}

/* ------------------------------

small debounce helper (not used in current code but kept)

------------------------------*/

void delay_debounce(void)

{

delay_ms(30);

}

/* End of file */